# import necessary libraries

from matplotlib import pyplot as plt

from scipy.optimize import root

from scipy.integrate import quad, trapezoid

from scipy.optimize import minimize

from pint import UnitRegistry

import numpy as np

import pandas as pd

u = UnitRegistry()

h = u.planck_constant

c = u.speed_of_light

k = u.boltzmann_constant

q = u.elementary_charge

eV = u.electron_volt

pi = np.pi

T_sun = 6000 * u.kelvin # temperature of the sun

T_cell = 300 * u.kelvin # temperature of the solar cell

f = np.pi/(14000) # solid angle factor

"""This python code is intended to model the detailed balance limit of efficiency for an intermediate band solar cell

The functions are as follows:

photon_flux - calculates the blackbody photon flux given some bandgap E_g, temperature of the material, and the chemical potential of that material

conduction_current_density - calculates the current density using the photon flux

power_input - calculates the total power input from the sun

efficiency - calculates the efficiency for a given bandgap energy and chemical potential

optimize - optimizes the chemical potential to find the maximum efficiency for each bandgap energy

main - main function to run the calculations and optionally plot and export results

plot_values - plots the efficiency vs intermediate bandgap energy

export - exports the results to a CSV file"""

def power_input(T_sun, cutoff, fX):

T_sun = T_sun.to('K').magnitude # Ensure temperature is in Kelvin

k = u.Quantity('boltzmann_constant').m_as('eV/K') # set the boltzmann constant in eV/K and remove units for calculation

integrand, error = quad(lambda x:(x**3/(np.exp((x / (k * T_sun)))-1)), 0 , cutoff) # Integrate the power input from 0 to a high value

P_in = fX * (pi/((h**3)*(c**2))) * integrand * u.Quantity('eV^4') # Calculate the total power input from the sun

return P_in  # Flux in photons/(m^2·s·m)

def photon_flux(min, max, T, mu, fX):

"""Calculate photon flux using Gauss-Legendre quadrature"""

T = T.m_as('K')

min = min.m_as('eV')

max = max.m_as('eV')

k = u.Quantity(1, 'boltzmann_constant').m_as('eV/K')

mu = mu.m_as('eV')

x = np.linspace(min, max, 1000)

y = x**2 / (np.exp((x - mu) / (k * T)) - 1)

y = np.maximum(y, 0)  # Prevent negative integrand values

integrand = trapezoid(y, x)

N = (fX * pi / ((h**3) * (c**2))) * integrand * eV**3

return N  # Ensure non-negative flux

def IB_current_density(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv, u_ci):

u_iv = u_cv - u_ci

if E_i < E_c:

flux_val = photon_flux(E_i, E_c, T_sun, 0 * eV, fX) - photon_flux(E_i, E_c, T_cell, u_iv * eV, 1)

flux_conduct = photon_flux(E_c, E_g, T_sun, 0 * eV, fX) - photon_flux(E_c, E_g, T_cell, u_ci * eV, 1)

else:

flux_val = photon_flux(E_i, E_g, T_sun, 0 * eV, fX) - photon_flux(E_i, E_g, T_cell, u_iv * eV, 1)

flux_conduct = photon_flux(E_c, E_i, T_sun, 0 * eV, fX) - photon_flux(E_c, E_i, T_cell, u_ci * eV, 1)

flux = (flux_val + flux_conduct).magnitude

if flux < 0:

return -flux

return flux

def CB_current_density(E_g, E_c, E_i, int_max, T_cell, T_sun, fX, u_cv, u_ci, u_iv):

"""Calculate the current density using the photon flux"""

conduct_flux = photon_flux(E_g, int_max * eV, T_sun, 0 * eV, fX) - photon_flux(E_g, int_max * eV, T_cell, u_cv * eV, 1)

if E_i  < E_c:

inter_flux_C = photon_flux(E_c, E_g, T_sun, 0 * eV, fX) - photon_flux(E_c, E_g, T_cell, u_ci * eV, 1)

inter_flux_V = photon_flux(E_i, E_c, T_sun, 0 * eV, fX) - photon_flux(E_i, E_c, T_cell, u_iv * eV, 1)

inter_flux = min(inter_flux_C, inter_flux_V)

else:

inter_flux_C = photon_flux(E_c, E_i, T_sun, 0 * eV, fX) - photon_flux(E_c, E_i, T_cell, u_ci * eV, 1)

inter_flux_V = photon_flux(E_i, E_g, T_sun, 0 * eV, fX) - photon_flux(E_i, E_g, T_cell, u_iv * eV, 1)

inter_flux = min(inter_flux_C, inter_flux_V)

flux = conduct_flux + inter_flux

J = q * flux

return J

def find_u_ci(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv):

def equation(u_ci):

return IB_current_density(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv, u_ci)

guess = max(0.5 * E_c.m_as('eV'), 0.5 * E_i.m_as('eV'))

zero = root(equation, (u_cv / E_g.m_as('eV')) * E_c.m_as('eV'), method='hybr', tol=1e-3,  options={'bounds': (0.01, 0.99 * E_c.m_as('eV'))})

u_ci = max(0, min(zero.x[0], E_c.m_as('eV')))  # Clip to [0, max_u_ci]

if u_ci == E_c.m_as('eV'):

u_ci = 0

return u_ci

def efficiency(x, E_g, T_cell, T_sun, int_max, fX, P_in):

"""Calculate dimensionless efficiency"""

E_i = x[0] * eV

E_c = E_g - E_i

u_cv = x[1]

u_ci = find_u_ci(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv)

u_iv = u_cv - u_ci

J = CB_current_density(E_g, E_c, E_i, int_max, T_cell, T_sun, fX, u_cv, u_ci, u_iv)

P_out = J * u_cv * eV / q

efficiency = P_out / P_in

if u_ci < 0 or u_iv < 0:

return 0

# print(f"E_i: {E_i}, u_cv: {u_cv}, u_ci: {u_ci}, u_iv: {u_iv}, Efficiency: {efficiency}")

if not 0 < efficiency.magnitude < 1:

return 0

return -efficiency.magnitude

def Optimize(E_g, int_max, T_cell, T_sun, fX):

"""Optimize chemical potentials for maximum efficiency"""

P_in = power_input(T_sun, int_max, fX)

# Initial guess

guess = [0.5 * E_g.m_as('eV'),

0.5 * E_g.m_as('eV')]

# Define bounds directly as a list of tuples

bounds = [(0, E_g.m_as('eV')),

(0, E_g.m_as('eV'))]

result = minimize(

efficiency,

args = (E_g, T_cell, T_sun, int_max, fX, P_in),

x0 = guess,

method='Powell',  # or 'L-BFGS-B'

bounds=bounds,

options={

'xtol': 1e-3,

'ftol': 1e-3,

'maxiter': 100,

}

)

if result.success:

Efficiency = -result.fun, result.x[0]

return Efficiency

else:

print(f"Optimization failed for E_g={E_g}")

return 0

def main(E_g, cplx, T_cell, T_sun, int_max, fX, Plot):

efficiencies = np.zeros(cplx)

E_i = np.zeros(cplx)

for i in range(cplx):

print(f"Optimizing for E_g = {E_g[i]:.2f} eV ({i+1}/{cplx})")

eff, e_i_val = Optimize(E_g[i], int_max, T_cell, T_sun, fX)

efficiencies[i] = eff

E_i[i] = e_i_val

print("\nOptimization complete!")

if Plot:

plot_values(E_g.m_as('eV'), E_i, efficiencies * 100)

return efficiencies

def plot_values(E_g, E_i, efficiencies):

"""Plot efficiency vs intermediate bandgap energy"""

plt.figure(figsize=(10, 6))

plt.plot(E_g, efficiencies, marker='o')

plt.xlabel('Intermediate Band Energy E_g (eV)')

plt.ylabel('Efficiency (%)')

plt.title('Efficiency of Intermediate Band Solar Cell')

for x, y, eg in zip(E_g, efficiencies, E_i):

plt.annotate(f'{eg:.2f}', (x, y), textcoords="offset points", xytext=(0,10), ha='center', fontsize=8)

plt.grid(True)

plt.show()

"""This portion of this code defines the parameters you wish to use"""

cplx = 9 # complexity of the calculations, higher values increase precision but also computation time

int_max = 6 # designate maximum energy to integrate to

E_g = np.linspace(0.6, 4, cplx) * eV

X = 4600 # concentration factor

Plot = True # change to true to plot results

fX = f * X # combined solid angle and concentration factor

results = main(E_g, cplx, T_cell, T_sun, int_max, fX, Plot)

# import necessary libraries

from matplotlib import pyplot as plt

from scipy.optimize import differential_evolution, minimize, root, shgo, dual_annealing, root_scalar

from scipy.integrate import quad

from pint import UnitRegistry

import numpy as np

import pandas as pd

import warnings

from scipy.integrate import IntegrationWarning

warnings.filterwarnings("ignore", category=RuntimeWarning)

warnings.filterwarnings("ignore", category=IntegrationWarning)

warnings.filterwarnings("ignore", category=UserWarning)

warnings.filterwarnings("ignore", category=DeprecationWarning)

warnings.filterwarnings("ignore", category=FutureWarning)

warnings.filterwarnings("ignore", category=Warning)

from scipy.stats import truncnorm

import csv

from concurrent.futures import ThreadPoolExecutor

# simplify unit registry

u = UnitRegistry()

# simplify constants

h = u.planck_constant

c = u.speed_of_light

k = u.boltzmann_constant

q = u.elementary_charge

eV = u.electron_volt

pi = np.pi

global uci

T_sun = 6000 * u.kelvin # temperature of the sun

T_cell = 300 * u.kelvin # temperature of the solar cell

"""This python code is intended to model the detailed balance limit of efficiency for an intermediate band solar cell

The functions are as follows:

photon_flux - calculates the blackbody photon flux given some bandgap E_g, temperature of the material, and the chemical potential of that material

power_input - calculates the total power input from the sun

IB_current_density - calculates the current density for the intermediate band

CB_current_density - calculates the current density for the conduction band

find_u_ci - finds the chemical potential between the intermediate and conduction bands

efficiency - calculates the efficiency for a given bandgap energy and chemical potential

optimize - optimizes the chemical potential to find the maximum efficiency for each bandgap energy

main - main function to run the calculations and optionally plot and export results

plot_values - plots the efficiency vs intermediate bandgap energy

export - exports the results to a CSV file"""

def power_input(T_sun, cutoff, fX):

# remove units from constants

T_sun = T_sun.to('K').magnitude

k = u.Quantity('boltzmann_constant').m_as('eV/K')

# integrate over blackbody spectrum

integrand, error = quad(lambda x:(x**3/(np.exp((x / (k * T_sun)))-1)), 0, cutoff)

# calculate power input

P_in = fX * (pi/((h**3)*(c**2))) * integrand * u.Quantity('eV^4')

# return power

return P_in  # Flux in photons/(m^2·s·m)

def photon_flux(min, max, T, mu, fX):

# remove units from constants

T = T.m_as('K')

min = min.m_as('eV')

max = max.m_as('eV')

k = u.Quantity(1, 'boltzmann_constant').m_as('eV/K')

mu = mu.m_as('eV')

# Suppress warnings and use more robust integration

with warnings.catch_warnings():

warnings.simplefilter("ignore")

flux, error = quad(lambda x: (x**2 / (np.expm1((x - mu) / (k * T)))), min, max)

N = (fX * pi / ((h**3) * (c**2))) * flux * eV**3

return N

def IB_current_density(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv, u_ci):

u_iv = u_cv - u_ci

if abs(E_i - E_c) < 1e-6 * eV:

return 0 * q

# if statememnt to choose limits of integration

if E_i < E_c:

flux_val = photon_flux(E_i, E_c, T_sun, 0 * eV, fX) - photon_flux(E_i, E_c, T_cell, u_iv, 1)

flux_conduct = photon_flux(E_c, E_g, T_sun, 0 * eV, fX) - photon_flux(E_c, E_g, T_cell, u_ci, 1)

else:

flux_val = photon_flux(E_i, E_g, T_sun, 0 * eV, fX) - photon_flux(E_i, E_g, T_cell, u_iv, 1)

flux_conduct = photon_flux(E_c, E_i, T_sun, 0 * eV, fX) - photon_flux(E_c, E_i, T_cell, u_ci, 1)

# determine total flux

flux = (flux_conduct - flux_val)

# Convert to current density (same units as CB_current_density)

J = q * flux

# return current density (not just flux)

return J

def CB_current_density(E_g, E_c, E_i, int_max, T_cell, T_sun, fX, u_cv, u_ci, rough):

u_iv = u_cv - u_ci

# conduction flux is simple to calculate

conduct_flux = photon_flux(E_g, int_max * eV, T_sun, 0 * eV, fX) - photon_flux(E_g, int_max * eV, T_cell, u_cv, 1)

# if statement to choose limits of integration, then choose minimum flux to ensure current balance

if E_i  <= E_c:

inter_flux_C = photon_flux(E_c, E_g, T_sun, 0 * eV, fX) - photon_flux(E_c, E_g, T_cell, u_ci, 1)

inter_flux_V = photon_flux(E_i, E_c, T_sun, 0 * eV, fX) - photon_flux(E_i, E_c, T_cell, u_iv, 1)

else:

inter_flux_C = photon_flux(E_c, E_i, T_sun, 0 * eV, fX) - photon_flux(E_c, E_i, T_cell, u_ci, 1)

inter_flux_V = photon_flux(E_i, E_g, T_sun, 0 * eV, fX) - photon_flux(E_i, E_g, T_cell, u_iv, 1)

if rough:

inter_flux = min(inter_flux_C, inter_flux_V)

else:

inter_flux = inter_flux_C

# calculate total flux

flux = conduct_flux + inter_flux

# calculate current density

J = q * flux

# return current density

return J

def find_u_ci(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv):

def equation(u_ci_arr, minimum, maximum):

u_ci = u_ci_arr[0]

IB = IB_current_density(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv, u_ci * eV)

return [IB.m_as('A/m^2')]

maximum = min(E_c, u_cv).m_as('eV')

minimum = u_cv.m_as('eV') - min(E_i, u_cv).m_as('eV')

sigma = np.sqrt(0.000168) * (maximum - minimum)

mean = 0.035812 * (maximum - minimum)

n = 20

# More diverse guesses: low, high, mid, uniform, linspace

guess_low = abs(minimum + np.random.normal(mean, sigma, n))

guess_high = abs(maximum - np.random.normal(mean, sigma, n))

guess_mid = np.random.uniform(minimum, maximum, n)

guess_lin = np.linspace(minimum, maximum, n)

guess = np.concatenate((guess_low, guess_high, guess_mid, guess_lin))

guess = guess.clip(minimum, maximum)

ordered = []

for i in guess:

ordered.append((i, equation([i], minimum, maximum)[0]))

ordered = sorted(ordered, key=lambda x: x[1])[:3]

found = []

for o in ordered:

sol = root(

equation,

x0=[o[0]],

args=(minimum, maximum),

method='hybr',

tol=1e-8)

if sol.success and minimum <= sol.x[0] <= maximum:

found.append((sol.x[0], sol.fun[0]))

if found and min(found, key=lambda x: x[1])[1]**2 < 1e-1:

u_ci = min(found, key=lambda x: x[1])[0]

return u_ci * eV, True

return 0 * eV, False

def efficiency(x, E_g, E_i, T_cell, T_sun, int_max, fX, P_in):

E_c = E_g - E_i

u_cv = x[0] * eV

u_ci, valid = find_u_ci(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv)

u_iv = u_cv - u_ci

if not valid:

return 0

if E_i <= 0 or E_c <= 0:

return 0

if not (0.01 < u_cv.m_as('eV') < 0.99 * E_g.m_as('eV')):

return 0

J = CB_current_density(E_g, E_c, E_i, int_max, T_cell, T_sun, fX, u_cv, u_ci, False)

P_out = J * u_cv / q

efficiency = P_out / P_in

if not 0 < efficiency < 1:

return 0

return -efficiency.magnitude

def Optimize(E_g, E_i, int_max, T_cell, T_sun, fX):

# Calculate power input once

P_in = power_input(T_sun, int_max, fX)

E_c = E_g - E_i

print(f"Optimizing for E_g={E_g.m_as('eV'):.2f}, E_i={E_i.m_as('eV'):.2f}...")

best_u_cv, valid = scan_u_cv(E_g, E_c, E_i, T_cell, T_sun, int_max, fX, P_in)

if not valid:

print(f"\033[91mScan failed for E_g={E_g.m_as('eV'):.2f}.\033[0m")

return -0.0

print(f"Scan guess for E_g={E_g.m_as('eV'):.2f}: u_cv={best_u_cv:.2f} eV, E_i={E_i.m_as('eV'):.2f} eV")

bounds = [(0.5 * best_u_cv, min(E_g.m_as('eV'), best_u_cv * 1.5))]

result = minimize(

lambda x: efficiency(x, E_g, E_i, T_cell, T_sun, int_max, fX, P_in),

x0=[best_u_cv],

bounds=bounds,

method='SLSQP',

tol=1e-1,

options={'maxiter': 200}

)

if result.fun < 0:

print(f"\033[92mOptimization successful for E_g={E_g.m_as('eV')}: Efficiency={-result.fun*100:.8f}%, E_i={E_i.m_as('eV'):.8f} eV, u_cv={result.x[0]:.8f} eV\033[0m")

return result.fun

else:

print(f"\033[91mOptimization failed for E_g={E_g.m_as('eV')}, using scan guess.\033[0m")

return -0.0  # or another fallback value

def scan_u_cv(E_g, E_c, E_i, T_cell, T_sun, int_max, fX, P_in):

# Use biased sampling for u_cv, emphasizing edges

n = 20  # Increase total samples for better coverage

u_cv_min = 0.3 * E_g.m_as('eV')

u_cv_max = 0.9 * E_g.m_as('eV')

mean = 0.55 * E_g.m_as('eV')

sigma = 0.15 * (u_cv_max - u_cv_min)

# Sample from Gaussian, clip to allowed range

guesses = np.random.normal(mean, sigma, n // 2)

guesses = np.clip(guesses, u_cv_min, u_cv_max)

# Add linspace points, biased toward edges (more points near min/max)

linspace_points = np.concatenate([

np.linspace(u_cv_min, u_cv_min + 0.1 * (u_cv_max - u_cv_min), n // 6),  # Near min

np.linspace(u_cv_max - 0.1 * (u_cv_max - u_cv_min), u_cv_max, n // 6),  # Near max

np.linspace(u_cv_min, u_cv_max, n // 3)  # Even spread

])

# Explicit edge guesses

edge_guesses = [u_cv_min, u_cv_max, (u_cv_min + u_cv_max) / 2]

guesses = np.concatenate([guesses, linspace_points, edge_guesses])

guesses = np.unique(np.clip(guesses, u_cv_min, u_cv_max))  # Remove duplicates and clip

best_eff = 0

best_u_cv = guesses[0]

for u_cv_val in guesses:

u_cv = u_cv_val * eV

u_ci, valid = find_u_ci(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv)

if not valid:

continue

J = CB_current_density(E_g, E_c, E_i, int_max, T_cell, T_sun, fX, u_cv, u_ci, False)

P_out = J * u_cv / q

eff = (P_out / P_in).magnitude

if 0 < eff < 1 and eff > best_eff:

best_eff = eff

best_u_cv = u_cv_val

print(f"Scan found best u_cv={best_u_cv:.8f} eV with efficiency={best_eff*100:.8f}%")

if best_eff == 0:

return 0, False

return best_u_cv, True

def optimize_point(args):

i, j, E_g, E_i, int_max, T_cell, T_sun, fX = args

if E_i < E_g:

result = Optimize(E_g * eV, E_i * eV, int_max, T_cell, T_sun, fX)

return (i, j, -result * 100)

else:

return (i, j, 0)

# Fix the main execution

if __name__ == '__main__':

X = 1

f = 1

fX = f * X

int_max = 10

cplx = 20

# Start with smaller range for testing

E_g = np.linspace(0.5, 4.5, cplx)

E_i = np.linspace(0.1, 4.5, cplx)

args_list = []

for i in range(cplx):

for j in range(cplx):

args_list.append((i, j, E_g[j], E_i[i], int_max, T_cell, T_sun, fX))

efficiencies = np.zeros((cplx, cplx))

print("Starting optimization...")

with ThreadPoolExecutor() as executor:

for i, j, eff in executor.map(optimize_point, args_list):

efficiencies[i, j] = eff

print(f"Max efficiency: {np.max(efficiencies):.2f}%")

# Plotting with contour lines

plt.figure(figsize=(10, 10))

# Create the color meshel='Efficiency (%)')

mesh = plt.pcolormesh(E_g, E_i, efficiencies, shading='auto', cmap='hot')

plt.xlabel('E_g (eV)')

plt.ylabel('E_i (eV)')

plt.title('Intermediate Band Solar Cell Efficiency Map with Contour Lines')

plt.colorbar(label='Efficiency (%)')

plt.show()

plt.grid(True, linestyle='--', alpha=0.5)

import numpy as np

import matplotlib.pyplot as plt

from pint import UnitRegistry

from scipy.integrate import quad

from scipy.optimize import curve_fit

import warnings

u = UnitRegistry()

h = u.planck_constant

c = u.speed_of_light

k = u.boltzmann_constant

q = u.elementary_charge

eV = u.electron_volt

pi = np.pi

def photon_flux(min, max, T, mu, fX):

T = T.m_as('K')

min = min.m_as('eV')

max = max.m_as('eV')

k = u.Quantity(1, 'boltzmann_constant').m_as('eV/K')

mu = mu.m_as('eV')

with warnings.catch_warnings():

warnings.simplefilter("ignore")

flux, error = quad(lambda x: (x**2 / (np.expm1((x - mu) / (k * T)))), min, max)

N = (fX * 2 * pi / ((h**3) * (c**2))) * flux * eV**3

return N

def IB_current_density(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv, u_ci):

u_iv = u_cv - u_ci

if E_i < E_c:

flux_val_abs = photon_flux(E_i, E_c, T_sun, 0 * eV, fX)

flux_val_emit = photon_flux(E_i, E_c, T_cell, u_iv, 1)

flux_conduct_abs = photon_flux(E_c, E_g, T_sun, 0 * eV, fX)

flux_conduct_emit = photon_flux(E_c, E_g, T_cell, u_ci, 1)

else:

flux_val_abs = photon_flux(E_i, E_g, T_sun, 0 * eV, fX)

flux_val_emit = photon_flux(E_i, E_g, T_cell, u_iv, 1)

flux_conduct_abs = photon_flux(E_c, E_i, T_sun, 0 * eV, fX)

flux_conduct_emit = photon_flux(E_c, E_i, T_cell, u_ci, 1)

flux = (flux_conduct_abs - flux_conduct_emit) + (flux_val_emit - flux_val_abs)

if flux_conduct_abs < flux_conduct_emit:

J = q * flux

return J

def tan_period(x, a, b, c, d):

return a * np.tan(b * (x + c)) + d

if __name__ == "__main__":

E_g = 3 * eV

E_i = 0.1 * eV

T_cell = 300 * u.kelvin

T_sun = 6000 * u.kelvin

fX = 1

u_cv = 2.5 * eV

plt.figure()

E_c = E_g - E_i

E_g_val = E_g.m_as('eV')

E_i_val = E_i.m_as('eV')

E_c_val = E_c.m_as('eV')

u_cv_val = u_cv.m_as('eV')

maximum = min(E_c_val, u_cv_val)

minimum = u_cv_val - min(u_cv_val, E_i_val)

u_ci = np.linspace(minimum + 1e-5, maximum - 1e-5, 100)

def equation(u_ci_val):

IB = IB_current_density(E_g, E_c, E_i, T_cell, T_sun, fX, u_cv, u_ci_val * eV)

return IB.m_as('A/m^2')

y = np.array([equation(u_ci_value) for u_ci_value in u_ci])

x = u_ci

# Initial guesses

a0 = y[np.argmax(np.abs(y - np.mean(y)))] - np.mean(y)

b0 = (pi) / (maximum - minimum)

c0 = -(maximum + minimum) / 2

d0 = np.mean(y)

print("Initial guesses:", a0, b0, c0, d0)

# Fit model

params, _ = curve_fit(

tan_period, x, y,

p0=[a0, b0, c0, d0]

)

print("Fitted parameters:", params)

# Predict and plot

x_fit = np.linspace(min(x), max(x), 1000)

y_fit = tan_period(x_fit, *params)

plt.plot(x, y, 'o', label='Data')

plt.plot(x_fit, y_fit, '-', label='Tangent Fit')

plt.legend()

plt.xlabel('x')

plt.ylabel('f(x)')

plt.title('Single-Period Tangent Fit')

plt.grid(True)

plt.show()

# Find all algebraic roots in the data range

a_fit, b_fit, c_fit, d_fit = params

root = (np.arctan(-d_fit / a_fit)) / b_fit - c_fit

print(minimum, maximum)

print("Algebraic root within data range:", root)